Packet Tracer - Connect a Router to a LAN

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.11.1 | 255.255.255.0 | N/A |
|  | S0/0/0 (DCE) | 209.165.200.225 | 255.255.255.252 | N/A |
| R2 | G0/0 | 10.1.1.1 | 255.255.255.0 | N/A |
|  | G0/1 | 10.1.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 209.165.200.226 | 255.255.255.252 | N/A |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 | NIC | 192.168.11.10 | 255.255.255.0 | 192.168.11.1 |
| PC3 | NIC | 10.1.1.10 | 255.255.255.0 | 10.1.1.1 |
| PC4 | NIC | 10.1.2.10 | 255.255.255.0 | 10.1.2.1 |

Objectives

Part 1: Display Router Information

Part 2: Configure Router Interfaces

Part 3: Verify the Configuration

Background

In this activity, you will use various show commands to display the current state of the router. You will then use the Addressing Table to configure router Ethernet interfaces. Finally, you will use commands to verify and test your configurations.

Note: The routers in this activity are partially configured. Some of the configurations are not covered in this course, but are provided to assist you in using verification commands.

Display Router Information

Display interface information on R1.

Note: Click a device and then click the CLI tab to access the command line directly. The console password is cisco. The privileged EXEC password is class.

Which command displays the statistics for all interfaces configured on a router?

Which command displays the information about the Serial 0/0/0 interface only?

Enter the command to display the statistics for the Serial 0/0/0 interface on R1 and answer the following questions:

What is the IP address configured on R1?

What is the bandwidth on the Serial 0/0/0 interface?

Enter the command to display the statistics for the GigabitEthernet 0/0 interface and answer the following questions:

What is the IP address on R1?

What is the MAC address of the GigabitEthernet 0/0 interface?

What is the bandwidth on the GigabitEthernet 0/0 interface?

Display a summary list of the interfaces on R1.

Which command displays a brief summary of the current interfaces, statuses, and IP addresses assigned to them?

____________________________________________________________________________________

Enter the command on each router and answer the following questions:

How many serial interfaces are there on R1 and R2?

How many Ethernet interfaces are there on R1 and R2?

________________________________________________________________________________

Are all the Ethernet interfaces on R1 the same? If no, explain the difference(s).

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Display the routing table on R1.

What command displays the content of the routing table?

Enter the command on R1 and answer the following questions:

How many connected routes are there (uses the C code)?

Which route is listed?

How does a router handle a packet destined for a network that is not listed in the routing table?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Configure Router Interfaces

Configure the GigabitEthernet 0/0 interface on R1.

Enter the following commands to address and activate the GigabitEthernet 0/0 interface on R1:

R1(config)# interface gigabitethernet 0/0

R1(config-if)# ip address 192.168.10.1 255.255.255.0

R1(config-if)# no shutdown

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

It is good practice to configure a description for each interface to help document the network information. Configure an interface description indicating to which device it is connected.

R1(config-if)# description LAN connection to S1

R1 should now be able to ping PC1.

R1(config-if)# end

%SYS-5-CONFIG_I: Configured from console by console

R1# ping 192.168.10.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.10.10, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 0/2/8 ms

Configure the remaining Gigabit Ethernet Interfaces on R1 and R2.

Use the information in the Addressing Table to finish the interface configurations for R1 and R2. For each interface, do the following:

Enter the IP address and activate the interface.

Configure an appropriate description.

Verify interface configurations.

Back up the configurations to NVRAM.

Save the configuration files on both routers to NVRAM. What command did you use?

_______________________________________________________________________________________

Verify the Configuration

Use verification commands to check your interface configurations.

Use the show ip interface brief command on both R1 and R2 to quickly verify that the interfaces are configured with the correct IP address and active.

How many interfaces on R1 and R2 are configured with IP addresses and in the “up” and “up” state?

____________________________________________________________________________________

What part of the interface configuration is NOT displayed in the command output?

What commands can you use to verify this part of the configuration?

____________________________________________________________________________________

Use the show ip route command on both R1 and R2 to view the current routing tables and answer the following questions:

How many connected routes (uses the C code) do you see on each router?

How many EIGRP routes (uses the D code) do you see on each router?

If the router knows all the routes in the network, then the number of connected routes and dynamically learned routes (EIGRP) should equal the total number of LANs and WANs. How many LANs and WANs are in the topology?

Does this number match the number of C and D routes shown in the routing table?

Note: If your answer is “no”, then you are missing a required configuration. Review the steps in Part 2.

Test end-to-end connectivity across the network.

You should now be able to ping from any PC to any other PC on the network. In addition, you should be able to ping the active interfaces on the routers. For example, the following should tests should be successful:

From the command line on PC1, ping PC4.

From the command line on R2, ping PC2.

Note: For simplicity in this activity, the switches are not configured; you will not be able to ping them.

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Display Router Information | Step 1a | 2 |  |
|  | Step 1b | 2 |  |
|  | Step 1c | 4 |  |
|  | Step 1d | 6 |  |
|  | Step 2a | 2 |  |
|  | Step 2b | 6 |  |
|  | Step 3a | 2 |  |
|  | Step 3b | 6 |  |
| Part 1 Total |  | 30 |  |
| Part 2: Configure Router Interfaces | Step 3 | 2 |  |
| Part 2 Total |  | 2 |  |
| Part 3: Verify the Configuration | Step 1a | 6 |  |
|  | Step 1b | 8 |  |
| Part 3 Total |  | 14 |  |
| Packet Tracer Score |  | 54 |  |
| Total Score (with bonus) |  | 100 |  |